A work in progress (WIP) documentation that contains all input files structure, all values for each parameter.

Solver Input File

{

"Solver":

[

{

"coordinateSystem" : <string>,

"type"              : <string>,

"startTime"         : <double>,

"endTime"           : <double>,

"timeStep"          : <double>,

"stoppingCriteria" : <double>,

“massMatrixType” : <boolean>,

“theta” : <double>

}

],

coodinateSystem : "AXIS" (Axisymmetric 2D) or “1D” or “2D” or “3D”

type : "transient" or “steady”,

// All variables for “solver”, below to be defined only when –> type = transient

startTime : Starting time for time step of a time dependent problems

endTime : End time step for a transient problems

timeStep : Each time step size

stoppingCriteria : Convergence criteria for the problem defined by user {usually very small value << 1}

massMatrixType : a boolean variable; 0 = Lumped Mass matrix ; 1 = Consistent Mass Matrix

theta : Generalized alpha time discretization algorithm; can take on of the 4 plausible values defined below

0 ->Forward Euler;  1 -> Backward Euler ; 2.0/3.0 -> Galerkin; 1.0/2.0 -> Crank-Nickolson

"Equation":

[

{

"name"          : <string>,		// Text Box in GUI

"type"            : <string>, 		// Drop Down menu in GUI

"meshFile"    : <string>,		// currently text box to be changed to drop down in future in GUI

"meshField"  : <string>,		// currently text box to be changed to drop down in future in GUI

"materialPropertyName" : <string>, // now text box to be changed to drop down in future in GUI

"FE_Shape"      : <string>, 	// Drop down menu in GUI

"GaussPoints"   : <int> 		// Text box

}

],

name : A user defined name as identifier for equation

type : Type of equation “heatTransfer”, “linearElastic”

meshFile : mesh file name

meshField : physical field name where the equation to be applied

materialPropertyName : Name of material property which correspond to this equation, to be used for linking 				the equations with material properties

FE_Shape : Type of finite element selected by user for descretization of the domain; to be as per problem

1D elements = “2NodeRod”, “3NodeRod”, “2NodeBeam”

2D elements = “3NodeTri”, “6NodeTri”, “4NodeQuad”, “9NodeQuad”

3D elements = “8NodeHexa”, “4NodeTetra”

GaussPoints : Number of Gauss points for each element

"MaterialProperty" :

[

{

"name" : "Elastic1",				// Text box in GUI

"type" : <string>,					// Drop down menu in GUI

"YoungsMod" : <double>,				// Text box in GUI

"NU" : <double>,					// Text box in GUI

“thermalConductivity” : <array> or <double>,	// Text boxes in GUI

"thickness": <double>,				// Text box in GUI

"rho" : <double>,					// Text box in GUI

“specificHeat” : <double>				// Text box in GUI

},

name : Text name given by user for identification of different types of material, also links with 		materialPropertyName in equations.

type : Material type currently = LinearElastic or HeatTransfer-iso or HeatTransfer-aniso

// Thickness to be defined only for 2D cases

thickness :  Domain thickness to be defined for only 2D problems.

// If type = LinearElastic

YoungsMod : Youngs modulus of stiffness based on material

NU : Poinson’s Ratio

// if Type =  HeatTransfer

thermalConductivity : double for isotropic material and array for anisotropic material

rho : Mass Density

specificHeat : Specific heat of material

Parts Input file

"parts":
[
	{
		"name" : <string>,		// text box in GUI
		"format" : <string>,		// Drop down menu in GUI
		"meshFileName": <string>	// Upload button in GUI
	}
]

name : Text name to identify the part {in case of multibody and big assemblies}
format : The mesh file format currently gmsh (.msh), in future hypermesh would be added
meshFileName : is the mesh file name with extension that contains the subpart of assemble or
Multi body

Boundary Input File

{

"neumann_BC":

[

{

"name": <string>,

"type": <string>,

"boundaryType": <string>,

"variable": <string>,

"meshFile": <string>,

"meshField": <string>,

"value": <scalar> ,

“values” : <vector> or <array of scalar or array of vectors>,

“filename”: <filePath>

},

name: The parameter can take any string. This name will help the user to distinguish between multiple boundary conditions.

type: This parameter can take “constant”, “temporal”, “constantNodalFileInput”, “temporalNodalFileInput”

boundaryType: This parameter can take “normalToBoundary”, “gradX”, “gradY”, “gradZ”

variable: This parameter can take “traction”, “convectiveHeatTransfer”, “heatFlux”, 	“HeatSourceSink”,“dispX”, “dispY”, “dispZ”, “velX”, “velY”, “velZ”, “temperature”

meshFile: meshFile name

meshField: physical name of the boundary to which the boundary condition is being applied

value: value can be scalar or vector, it will be scalar for the boundaryType gradX, gradY and gradZ. It will ve vector the boundaryType normalToBoundary. This parameter is not used for the parameter type *FileInput. Instead include the fileName parameter.

fileName: This parameter is used for the parameter type *FileInput. This contain the file path. The data inside the file can be scalar or vector and will in the order of the node in the physical field.

"dirichlet_BC":

[

{

"name": <string>,

"type": <string>,

"variable": <string>,

"meshFile": <string>,

"meshField": <string>,

"value": <scalar>,

“values” : <array>,

“filename”: <filePath>

},

name: The parameter can take any string. This name will help the user to distinguish between multiple boundary conditions.

type: This parameter can take “constant”, “temporal”, “constantNodalFileInput”, “temporalNodalFileInput”

variable: This parameter can take “dispX”, “dispY”, “dispZ”, “velX”, “velY”, “velZ”, “temperature”, “bodyForce”

meshFile: meshFile name

meshField: physical name of the boundary to which the boundary condition is being applied

value: value will be scalar or array, it will be scalar for the type constant. It will be array for the type temporal. This parameter is not used for the parameter type *FileInput. Instead include the fileName parameter.

values: If variable == “bodyForce” then, it’s the array containing “bodyForce” values of each nodal point in domain, in the form of [bodyForceX, bodyForceY, bodyForceZ]

fileName: This parameter will be scalar used for the parameter type *FileInput. This contain the file path. The data inside the file will be scalar and will in the order of the node in the physical field.

"initial_BC":

[

{

"name": <string>,

"type": <string>,

"variable": <string>,

"meshFile": <string>,

"meshField": <string>,

"value": <scalar/array>

“filename”: <filePath>

},

name: The parameter can take any string. This name will help the user to distinguish between multiple boundary conditions.

type: This parameter can take “constant {1}”, “nodalFileInput {2}”

variable: This parameter can take “dispX”, “dispY”, “dispZ”, “velX”, “velY”, “velZ”, “temperature”, “bodyForce”

meshFile: meshFile name

meshField: physical name of the boundary to which the boundary condition is being applied

value: value will be scalar it will be scalar for the type constant. This parameter is not used for the parameter type nodalFileInput. Instead include the fileName parameter.

values: It is the array containing “bodyForce” values (if any) at each nodal point in domain, in the form of [bodyForceX, bodyForceY, bodyForceZ]

fileName: This parameter will be scalar used for the parameter type nodalFileInput. This contain the file path. The data inside the file will be scalar and will in the order of the node in the physical field.

Examples of JSON input files

Solver Input File

{

"Solver":

[

{

"coordinateSystem"  : "2D”,

"type"              : "transient",

"startTime"         : 0,

"endTime"           : 100,

"timeStep"          : 0.01,

"stoppingCriteria" : 1e-6,

“massMatrixType” : 0 or 1,

“theta” : 1

}

],

"Equation":

[

{

"name"          : "equation1",

"type"            : "heatTransfer",

"meshFile"    : "test.msh",

"meshField"  : "test",

"materialPropertyName" : "Elastic1",

"FE_Shape"      : "3NodeTri",

"GaussPoints"   : 7

}

],

"MaterialProperty" :

[

{

"name" : "Elastic1",

"type" : "LinearElastic",

"YoungsMod" : 450,

"NU" : 0.3,

"thickness": 0.2

},

{

"name" : "heatTransfer",

"type" : "HeatTransfer-iso",

“thermalConductivity” : [10.5, 0.5, 0.5, 15.6]

“specificHeat” : 45.2,

"rho" : 35,

"thickness": 0.5

}

]

}